|  | Residential Code - 2002 Edition - R502.5 Allowable girder spans. The allowable spans of girders fabricated of dimension lumber shall not exceed the values set forth in Tables R502.5(1) and R502.5(2). TABLE R502.5(1) GIRDER AND HEADER SPANS FOR INTERIOR AND EXTERIOR LOAD CONDITIONS 20 psf Roof Live Load Building Width (ft.). 20; 28; 36; 20; 28; 36; 20; 28; 36; 20; 28; 36. Maximum Header/Girder Spans (ft-in), for Common Lumber Species1. Douglas-Fir-Larch; Hem-Fir; Southern Pine; Spruce-Pine-Fir. Headers Supporting; Size; ft-in; ft-in; ft-in; ft-in; ft-in; ft-in; ft-in; ft-in; ft-in; ft-in; ft-in; ft-in. Roof and Ceiling; 2-2x4; 3-9; 3-5; 3-1; 3-6; 3-2; 2-11; 3-9; 3-5; 3-2; 3-7; 3-3; 3-0. 2-2x6; 5-10; 5-0; 4-6; 5-6; 4-11; 4-5; 5-11; 5-3; 4-8; 5-8; 5-0; 4-5. 2-2x8; 7-4; 6-5; 5-8; 7-2; 6-2; 5-6; 7-8; 6-8; 5-11; 7-3; 6-3; 5-7. 2-2x10; 9-0; 7-9; 7-0; 8-9; 7-7; 6-9; 9-4; 8-1; 7-3; 8-10; 7-8; 6-10. 2-2x12; 10-5; 9-0; 8-1; 10-2; 8-9; 7-10; 10-10; 9-5; 8-5; 10-3; 8-11; 8-0. 3-2x8; 8-11; 8-0; 7-2; 8-4; 7-7; 6-11; 8-11; 8-1; 7-5; 8-6; 7-9; 7-0. 3-2x10; 11-3; 9-9; 8-9; 10-7; 9-6; 8-6; 11-4; 10-2; 9-1; 10-10; 9-7; 8-7. 3-2x12; 13-1; 11-4; 10-1; 12-8; 11-0; 9-10; 13-7; 11-9; 10-6; 12-11; 11-2; 10-0. 4-2x8; 9-10; 8-11; 8-3; 9-2; 8-4; 7-9; 9-10; 8-11; 8-3; 9-4; 8-6; 7-11. 4-2x10; 12-6; 11-3; 10-1; 11-8; 10-7; 9-9; 12-6; 11-4; 10-6; 11-11; 10-10; 9-11. 4-2x12; 15-1; 13-1; 11-8; 14-2; 12-8; 11-4; 15-2; 13-7; 12-2; 14-7; 12-11; 11-6. Roof, Ceiling, and One Center-Bearing Floor; 2-2x4; 3-4; 2-11; 2-7; 3-1; 2-10; 2-7; 3-4; 3-0; 2-9; 3-2; 2-10; 2-7. 2-2x6; 4-10; 4-3; 3-10; 4-8; 4-2; 3-9; 5-0; 4-5; 4-0; 4-9; 4-2; 3-9. 2-2x8; 6-2; 5-5; 4-10; 6-0; 5-3; 4-9; 6-5; 5-7; 5-1; 6-1; 5-4; 4-9. 2-2x10; 7-6; 6-7; 5-11; 7-3; 6-5; 5-9; 7-10; 6-10; 6-2; 7-5; 6-6; 5-10. 2-2x12; 8-8; 7-8; 6-11; 8-5; 7-5; 6-8; 9-1; 7-11; 7-2; 8-7; 7-6; 6-9. 3-2x8; 7-8; 6-9; 6-1; 7-4; 6-7; 5-11; 7-11; 7-0; 6-4; 7-6; 6-8; 6-0. 3-2x10; 9-5; 8-3; 7-5; 9-1; 8-0; 7-3; 9-9; 8-7; 7-9; 9-3; 8-1; 7-4. 3-2x12; 10-11; 9-7; 8-7; 10-7; 9-3; 8-5; 11-4; 9-11; 9-0; 10-9; 9-5; 8-6. 4-2x8; 8-8; 7-9; 7-0; 8-1; 7-5; 6-10; 8-8; 7-11; 7-4; 8-4; 7-7; 6-11. 4-2x10; 10-10; 9-6; 8-7; 10-4; 9-3; 8-4; 11-1; 9-11; 8-11; 10-7; 9-5; 8-6. 4-2x12; 12-7; 11-0; 9-11; 12-3; 10-9; 9-8; 13-1; 11-6; 10-4; 12-5; 10-11; 9-10. Roof, Ceiling, and One; 2-2x4; 2-11; 2-6; 2-3; 2-10; 2-5; 2-2; 3-0; 2-7; 2-4; 2-10; 2-5; 2-2. Clear-Span Floor; 2-2x6; 4-3; 3-8; 3-3; 4-1; 3-7; 3-2; 4-5; 3-10; 3-5; 4-2; 3-7; 3-3. 2-2x8; 5-4; 4-7; 4-2; 5-2; 4-6; 4-0; 5-7; 4-10; 4-4; 5-3; 4-7; 4-1. 2-2x10; 6-6; 5-8; 5-1; 6-4; 5-6; 4-11; 6-9; 5-10; 5-3; 6-5; 5-7; 5-0. 2-2x12; 7-7; 6-7; 5-10; 7-4; 6-4; 5-8; 7-10; 6-10; 6-1; 7-5; 6-5; 5-9. 3-2x8; 6-8; 5-9; 5-2; 6-6; 5-7; 5-0; 6-11; 6-0; 5-5; 6-7; 5-8; 5-1. 3-2x10; 8-2; 7-1; 6-4; 7-11; 6-10; 6-2; 8-6; 7-4; 6-7; 8-1; 7-0; 6-3. 3-2x12; 9-6; 8-2; 7-4; 9-2; 8-0; 7-1; 9-10; 8-6; 7-8; 9-4; 8-1; 7-3. 4-2x8; 7-9; 6-8; 6-0; 7-5; 6-6; 5-10; 7-11; 6-11; 6-3; 7-7; 6-7; 5-11. 4-2x10; 9-5; 8-2; 7-4; 9-2; 7-11; 7-1; 9-10; 8-6; 7-7; 9-3; 8-1; 7-2. 4-2x12; 10-11; 9-6; 8-6; 10-7; 9-2; 8-3; 11-4; 9-10; 8-10; 10-9; 9-4; 8-4. Roof, Ceiling, and Two Center-Bearing Floors; 2-2x4; 2-9; 2-5; 2-2; 2-8; 2-4; 2-1; 2-10; 2-6; 2-3; 2-8; 2-4; 2-1. 2-2x6; 4-0; 3-6; 3-2; 3-11; 3-5; 3-1; 4-2; 3-8; 3-3; 3-11; 3-5; 3-1. 2-2x8; 5-1; 4-5; 4-0; 4-11; 4-4; 3-11; 5-3; 4-7; 4-2; 5-0; 4-5; 3-11. 2-2x10; 6-2; 5-5; 4-11; 6-0; 5-3; 4-9; 6-5; 5-8; 5-1; 6-1; 5-4; 4-10. 2-2x12; 7-2; 6-4; 5-8; 7-0; 6-1; 5-6; 7-6; 6-7; 5-11; 7-1; 6-2; 5-7. 3-2x8; 6-4; 5-7; 5-0; 6-2; 5-5; 4-10; 6-7; 5-9; 5-3; 6-3; 5-6; 4-11. 3-2x10; 7-9; 6-10; 6-1; 7-6; 6-7; 5-11; 8-1; 7-1; 6-4; 7-8; 6-8; 6-0. 3-2x12; 9-0; 7-11; 7-1; 8-9; 7-8; 6-11; 9-4; 8-2; 7-5; 8-10; 7-9; 7-0. 4-2x8; 7-4; 6-5; 5-9; 7-1; 6-3; 5-8; 7-8; 6-8; 6-0; 7-3; 6-4; 5-8. 4-2x10; 8-11; 7-10; 7-1; 8-8; 7-8; 6-10; 9-4; 8-2; 7-4; 8-10; 7-9; 7-0. 4-2x12; 10-5; 9-1; 8-2; 10-1; 8-10; 8-0; 10-10; 9-6; 8-6; 10-3; 9-0; 8-1. Roof, Ceiling, and Two; 2-2x4; 2-3; 1-11; 1-9; 2-2; 1-11; 1-7; 2-4; 2-0; 1-10; 2-2; 1-10; 1-7. Clear-Span Floors; 2-2x6; 3-3; 2-10; 2-6; 3-2; 2-9; 2-6; 3-5; 2-11; 2-8; 3-3; 2-10; 2-5. 2-2x8; 4-2; 3-7; 3-3; 4-0; 3-6; 3-1; 4-4; 3-9; 3-4; 4-1; 3-6; 3-2. 2-2x10; 5-1; 4-5; 3-11; 4-11; 4-3; 3-10; 5-3; 4-7; 4-1; 5-0; 4-4; 3-10. 2-2x12; 5-10; 5-1; 4-7; 5-8; 4-11; 4-5; 6-1; 5-4; 4-9; 5-9; 5-0; 4-6. 3-2x8; 5-2; 4-6; 4-0; 5-1; 4-4; 3-11; 5-5; 4-8; 4-2; 5-1; 4-5; 4-0. 3-2x10; 6-4; 5-6; 4-11; 6-2; 5-4; 4-9; 6-7; 5-9; 5-1; 6-3; 5-5; 4-10. 3-2x12; 7-4; 6-4; 5-8; 7-2; 6-2; 5-6; 7-8; 6-8; 5-11; 7-3; 6-3; 5-7. 4-2x8; 6-0; 5-2; 4-8; 5-10; 5-1; 4-6; 6-3; 5-5; 4-10; 5-11; 5-1; 4-7. 4-2x10; 7-4; 6-4; 5-8; 7-1; 6-2; 5-6; 7-7; 6-7; 5-11; 7-3; 6-3; 5-7. 4-2x12; 8-6; 7-4; 6-7; 8-3; 7-2; 6-5; 8-10; 7-8; 6-10; 8-4; 7-3; 6-6. 1 The number of jack studs required at each end of the header shall be determined from Table 5052.5(1)b. Table R5052.5(1)b JACK STUD REQUIREMENTS FOR HEADERS IN EXTERIOR LOADBEARING WALLS Header Width. 3" (2-2x); 4.5" (3-2x); 6" (4-2x). Headers Supporting; Header Span (ft.); Number of Jack Studs Required at Each End of the Header1,2. Roof and Ceiling; 2; 1; 1; 1. 4; 1; 1; 1. 6; 2; 1; 1. 8; 2; 2; 1. 10; 3; 2; 2. 12; 3; 2; 2. 14; 4; 3; 2. 16; 4; 3; 2. Roof, Ceiling,; 2; 1; 1; 1. and One Floor; 4; 2; 1; 1. (Center Bearing); 6; 2; 2; 1. 8; 3; 2; 2. 10; 4; 3; 2. 12; 4; 3; 2. 14; 5; 3; 3. 16; 6; 4; 3. Roof, Ceiling,; 2; 1; 1; 1. and One Floor; 4; 2; 2; 1. (Clear Span); 6; 3; 2; 2. 8; 4; 3; 2. 10; 5; 3; 3. 12; 6; 4; 3. 14; 6; 4; 3. 16; 7; 5; 4. Roof, Ceiling,; 2; 1; 1; 1. and Two Floors; 4; 2; 2; 1. (Center Bearing); 6; 3; 2; 2. 8; 4; 3; 2. 10; 5; 3; 3. 12; 6; 4; 3. 14; 7; 5; 4. 16; 8; 5; 4. Roof, Ceiling,; 2; 2; 1; 1. and Two Floors; 4; 3; 2; 2. (Clear Span); 6; 5; 3; 3. 8; 6; 4; 3. 10; 7; 5; 4. 12; 9; 6; 5. 14; 10; 7; 5. 16; 12; 8; 6. 1 Jack stud requirements are based on a roof span (W) of 36’. For other roof spans, the tabulated number of jack studs required at each end of the header shall be multiplied by (W/48+ 1/4). 2 Where the number of jack studs equal 1, the header shall be permitted to be supported by a framing anchor attached to the full-height wall stud.. |
| --- | --- |