Qty	Craft	Hours	Unit	Material	Labor	Equipment	Total

__________________________________________________________________________

Roofing, cost per in-place square (Sq), removed using hand tools (1 square = 100 square feet)

Built-up, 5 ply

Removal of built-up roofing

(2.5 Sq per CY and 250 lbs per Sq)

1.00	BL@	1.500	SQ	0.00	90.00	0.00	90.00

Expanded polystyrene roof insulation

2", R-14.3, C-0.07

1.00	R3@	.6840	SQ	80.75	41.04	0.00	121.79

CSPE roofing

CSPE (Chloro-sulfonated Polyethylene) .045" single ply membrane

1.00	R3@	.7430	SQ	149.94	44.58	0.00	194.52

Add for mechanical fasteners

6" long, 12" OC at laps

200.00	--@	.0000	Ea	135.24	0.00	0.00	135.24

__________________________________________________________________________

Total Manhours, Material, Labor, and Equipment:

2.9	365.93	175.62	0.00	541.55

Total Only (Subcontract) Costs:	0.00

Subtotal:	541.55

15.00% Overhead:	81.23

5.00% Contingency:	31.14

15.00% Profit:	98.09

Estimate Total:	752.01

7.00% Tax on Materials:	25.62

0.00% Tax on Labor:	0.00

0.00% Tax on Equipment:	0.00

0.00% Tax on Total Only Costs:	0.00

0.00% Tax on the Contract Price	0.00

Grand Total:	$777.63  /100SF