Qty	Craft	Hours	Unit	Material	Labor	Equipment	Total

__________________________________________________________________________

Exterior deck joist system.  Includes 5% waste.  Complete joist system.  Does not include decking, railing, or stairs.  Includes footings, excavation for footings, column stirrups and connectors, posts, and beams on outside edge of deck, connection with joist hangers against house, and joists.

2" x 12" joists

1.00	6C@	.0830	SF	3.68	3.32	0.00	7.00

Exterior decking.  Includes 2" x 4" decking lumber, rust-resistant screws or nails, and installation labor. Deduct 8% for 5/4 treated wood decking (decking that is 1-1/4" thick instead of 1-1/2" thick). Includes 5% waste.

2" x 6" redwood exterior deck, construction heart

1.00	6C@	.0370	SF	4.35	1.48	0.00	5.83

Exterior deck railing.  Includes 2" x 6" top rail, often routed with a bull-nose, 2" x 2" balusters placed approximately 4" on center, fasteners, and installation labor. Does not include posts. Also includes 5% waste.

redwood construction heart

1.00	6C@	.2750	LF	9.73	11.00	0.00	20.73

Exterior deck stairs with railing, per step.  Includes up to three stringers made from 2" x 12" lumber. Each step has up to three 2" x 4" treads. Balusters are 2" x 2" placed approximately 4" on center. Balusters attach to side stringers on the bottom and a top rail at the top. Top rail is 2" x 6" often routed with a bullnose edge. Does not include foundation system for bottom of stairs. Includes 5% waste.

3' wide exterior deck stairs

construction heart redwood

1.00	6C@	.5870	Ea	40.18	23.48	0.00	63.66

Exterior deck landing.  Includes 2" x 10" joist system with 2" x 6" board decking, beams, posts, stirrups, footings, excavation for footings, all fasteners, and installation labor. Includes 5% waste.

construction heart decking

1.00	6C@	.0740	SF	13.72	2.96	0.00	16.68

Deck cover framing  Can also be referred to as a pergola. Conventionally framed wood cover joists. Per SF of area covered. Figures in parentheses indicate board feet per square foot of ceiling including end joists, header joists, and 5% waste. Costs shown are based on a job with 200 SF of area covered and include normal waste. Add the cost of posts and finish roof.

Deck covers, 2" x 6"

16" centers (.88 BF per SF)

1.00	B1@	.0200	SF	0.49	0.80	0.00	1.29

Add for posts (for pergola) to support conventionally framed wood cover joists. Posts are usually required at each corner. Includes one sack of concrete but no excavation

6" x 6" pressure treated, 10' long

1.00	B1@	.7500	Ea	27.64	30.00	0.00	57.64

_________________________________________________________________________

Total Manhours, Material, Labor, and Equipment:

1.8	99.79	73.04	0.00	172.83

Subtotal:	172.83

20.00% Overhead:	34.57

10.00% Contingency:	20.74

20.00% Profit:	45.63

Estimate Total:	273.77

7.00% Tax on Materials:	6.99

Grand Total:	280.76