PROJECT:  LOT 19 BINGHAM RIDGE

DOCUMENT:

DESCRIPTION OF MATERIALS

DATE:			   March 20, 2006

| LOT NO.: | 19 |
| --- | --- |

| OWNER: |  |
| --- | --- |

| 1. EXCAVATION: |  |
| --- | --- |
| Bearing soil | .Undisturbed earth |

| 2. FOUNDATIONS: |  |
| --- | --- |
| FOOTINGS: |  |
| Strength concrete | 3000 psi |
| Reinforcing Building site prep Excavation Size Site Conditions Topsoil | 2 #4 rebar, con’t Building site to be cleared and leveled Excavation to be trench style. Crawlspace drains (2) are to be placed below footing prior to placement of concrete. 20” x 10” continuous Existing topsoil/organic material to be removed prior to footing construction. |
| FOUNDATION WALL: |  |
| Material Sealed Crawlspace | 8" CMU. Stucco coating shall be applied directly over CMU, sloped to drain to outer edge of the footing. Interior of Cmu foundation wall to be faced with Dow Thermax rigid insulation; min of 1 1/2” thickness, bonded to CMU with Dow approved adhesive. Thermax to rest upon top of footing, and show 1 _” termite inspection reveal along top edge, continuous. 14 mil vapor barrier, with sealed joints and edges, to be laid over entire crawlspace area. Vapor barrier to be lapped up sides and piers approx 1 foot, and adhered to the face of the Thermax with approved mastic. A mechanical dehumidifier to be installed upon completeion of sheet rock to insure proper extraction of ecxcess moisture. |
| Waterproofing | Tuff n Dry; Deco, or equal, applied as per manufacturer’s specifications |
| Footing drains | 4" Perf. Plastic, drained to daylight by gravity. Pipe is to be embedded in washed stone and filtered where necessary to prevent silt infiltration. |
| Termite protection | Durisban TC; Borate; or approved termiticide application. |
| GIRDERS: |  |
| Material | SYP #2, or steel I-Beam, as indicated on the engineered drawings |
| Size | as indicated on engineered drawings |
| Sills: | 2 x 6, typical, sealed |
| Material | P.T. SYP |
| ADDITIONAL INFORMATION: | Steel and LVL girders are to be used as indicated on the drawings. |

| 3. CHIMNEYS: |  |
| --- | --- |
| PREFABRICATED CHIMNEY: |  |
| Flue lining: |  |
| Flue size | 8” as supplied by Majestic or equal |
| Additional information |  |
| MASONRY CHIMNEY | N/A |

| 4. FIREPLACES: | Majestic CVR36” or equal |
| --- | --- |
| Type of Construction | Metal , manufactured |
| Fuel type | Gas burning |
| Fireplace facing | Multicolor slate |
| Lining | Refractory-lined firebox |
| Hearth | Sandstone slab in Hearth Room and Dining Room |
| Glass doors | Yes – matte black –GDKBK36 |

| 5. EXTERIOR WALLS: |  |
| --- | --- |
| Wood frame:, | 2x6 stud @16”o.c. |
| Grade/species, | West Coast |
| Sheathing: | _” Dow Styrofoam Duramate Plus, with all horizontal and vertical joints taped and sealed. Note: all exterior walls to feature a let in style bracing method, so that insulated sheathing may be installed continuously. |
| Thickness | _” |
| Width | 4' x 8' |
| Siding: Grade Soffits, Fascia & Corner Boards | Hardie Plank Miratech composite trim or equivalent |
| Type | Smooth face, or wood grain |
| Exposure | 7” |
| Fastening | Blind nail pattern, galv fasteners |
| Gable wall construction | 2x4 @ 16” o.c.; w/ Hardie Side Wall Shingles, or Cedar sidewall shingles, as called out on plans |
| Additional information Head details Arches Ground line detail | Exterior trim details per plan Stone veneer per plan, dry stack Tennesse Sandstone |

| 6. FLOOR FRAMING: |  |
| --- | --- |
| WOOD: | #2 SYP |
| Joists Note | 2x10, except as noted on drawings |
| CONCRETE: | Floor of garage Exterior flat work as shown on site plan |
| Slab fill | 4” washed stone |
| Ground supported | Yes |
| Thickness | 4" |
| Membrane | 6 mil PVC |
| Washed stone, thickness | Backfill |
| Finish: | Slab: steel trowel finish |

| 7. SUBFLOORING: |  |
| --- | --- |
| Generally | 3/4" T&G Plywood; OSB |
| Additional information | All subfloor nailed and glued to joists. Underlayment grade plywood under areas of areas of hardwood; underlayment grade OSB under carpeted areas. |
| 8. FINISH FLOORING: |  |
| WOOD |  |
| Locations | Living room; Dining; Kitchen; Entry foyer; Powder Rm, Hearth room |
| Grade/Species | Common Red Oak Common Red Oak with Brazilian Cherry border in Living |
| Thickness | _” |
| Width | 2 _” |
| Building paper | #15 felt |
| Finish | 3 coats polyurethane |
| VINYL: |  |
| Locations |  |
| Material description |  |
| Total allowance |  |
| CARPET: |  |
| Locations | 1st floor office; Master bedroom/closet; 2nd floor hall; bedrooms #2, #3 and #4 |
| Material description | Carpet and pad |
| Total allowance | $30 per yard installed |
| CERAMIC TILE: |  |
| Locations Material description | Master bath, Bath #3, Bath #4, Utility, Craft Master Bath: Poreclain Hampton 12”x12” |
| Material description Material description | Bath #3, Bath #4: Ceramic 3 _” off white octagon with cobalt blue mosaic Utility, Craft: Ceramic Sahara Beige 12”x12” |
| Total allowance |  |
| OTHER FLOORING: |  |
| Locations | Foyer |
| Material description | Inlaid Multicolor Slate 12”x12” with Common Red Oak border |
| Locations Material description | Living Room by South Wall in Area Shown on Plan Inlaid Multicolor Slate 12”x12” |

| 9. PARTITION FRAMING: |  |
| --- | --- |
| Wood,grade,species | SPF stud grade & #2 |
| Size and Spacing | 2x4 |

| 10. CEILING FRAMING: |  |
| --- | --- |
| Joists (with band) | 2X8 and 2X10 AS SPECIFIED ON PLAN |
| Beams | As specified |

| 11. ROOF FRAMING: |  |
| --- | --- |
| Rafters, Main | 2x8 SPF |
| Spacing | 16” o.c. |
| Rafters, secondary |  |
| Spacing |  |
| Rafters, porches |  |
| Spacing |  |
| Trusses |  |
| Edge treatment Sub face Facia | 2x6 sub facia |
| Attic Space, describe |  |

| 12. ROOFING: |  |
| --- | --- |
| Sheathing | 7/16” Nexford Solar Board or equivalent |
| Finish roof material | 25yr Horizon Shingles or equivalent |
| Additional information | WinterGuard adhesive underlayment under all required roof penetrations |
| Membrane systems |  |

| 13. GUTTERS AND DOWNSPOUTS: |  |  |
| --- | --- | --- |
| Material | 4” Aluminum |  |
| Downspouts connected to: | 4” connected to 4” corrugated piping, buried |  |

| 14. LATH AND PLASTER: |  |
| --- | --- |
| Interior Walls: |  |
| Drywall | All interior surfaces to be _” drywall, smooth surface unless otherwise noted |
| Ceilings |  |
| Other |  |

15. DECORATING: See finish schedule.

| 16. INTERIOR DOORS & TRIM: |  |  |
| --- | --- | --- |
| DOORS: |  |  |
| Type:Location Type:Location Other Trim | 2 Panel Stain Grade Pine:Downstairs. Bi-Fold Mirror Door Mstr.Closet 2 panel, semi-solid paint grade: Upstairs Posts wrapped in Oak. Strain Grade Trim applied to soffit in Dining,Hearth Media Rooms. 4” Stain Grade Molding @ Ceiling in Office. Stair Rails/Newels & Balusters-Stain Grade Oak. |  |
| Thickness | 1 3/8” |  |
| Jambs | As supplied by factory |  |
| Non typical units |  |  |
| CASINGS: |  |  |
| Material Material | Sheetrock Returns @Windows 3 _ “ Smooth Rails Sides & 5 _” Craftsman Heading @ Doors |  |
| Size |  |  |
| Style | Maple Sills & Aprons @ Windows |  |
| BASEMOLD: |  |  |
| Material | Paint Grade 5 _” Base |  |
| Size | 5 _” |  |
| Style | Colonial |  |
| Shoe strips | Oak @ hardwood floors, paint grade @ tile |  |
| 17. WINDOWS: |  |  |
| Type | Casement & DoubleHung & Horizontal slider |  |
| Glazing | Low-E(2) @ East,North, and West elevations; Low-E(1) or clear IG @ South |  |
| Make | Marvin Integrity or Equivalent |  |
| Flashings |  |  |
| Screens | @all operative windows |  |
| Special Units |  |  |
| Exterior trim | Nailing fins |  |
| Interior Trim | Custom Hardwood Window Sills, Wrapped Openings |  |
| Snap in dividers | No; grilles between glass |  |
| Fixed Units |  |  |

| 18. ENTRANCES AND EXTERIOR DETAILS: |  |  |
| --- | --- | --- |
| MAIN ENTRANCE DOOR: |  |  |
| Material | Insulated Fiberglass with Low-E Glass |  |
| Thickness | 1 _” |  |
| Frame material | Factory supplied |  |
| Allowance |  |  |
| SECONDARY ENTRANCE: |  |  |
| Material | Insulated Fiberglass with Low-E Glass |  |
| Thickness | 1 _” |  |
| Frame material | Factory supplied |  |
| Allowance |  |  |
| SECONDARY ENTRANCE: |  |  |
| Material | ThermaTrue Premium Steel entry @ Garage; 20 minute fire rating |  |
| Thickness | 1 _” |  |
| Frame material | Factory supplied |  |
| Allowance |  |  |
| SECONDARY ENTRANCE: |  |  |
| Material | Garage doors-steel-coachmen style-Clopay fully insulated 8*7 |  |
| Thickness |  |  |
| Frame material |  |  |
| Allowance |  |  |
| SECONDARY ENTRANCE: |  |  |
| Material |  |  |
| Thickness |  |  |
| Frame material |  |  |
| Allowance |  |  |
| SECONDARY ENTRANCE: |  |  |
| Material |  |  |
| Thickness |  |  |
| Frame material |  |  |
| Allowance |  |  |

| 19. CABINETS ETC. |  |
| --- | --- |
| Kitchen |  |
| Cabinets | Custom Maple |
| Mfr. |  |
| Counter tops | Granite |
| Bathroom |  |
| Cabinets | Custom Maple/cherry or teak |
| Mfr. |  |
| Counter tops | Cultured marble in master bath and baths #3, #4, Soapstone in powder room, Laminate in craft room and Utility |
| Other cabinets –utility/craft | Maple |
| Built-ins | Hearth Room-Maple per plan; Living Room Media Center-Cherry per plan |
| Total cabinet pull allowance | $2/pull |

| 20a. STAIRS: |  |
| --- | --- |
| Tread | White oak |
| Risers | Stain Grade Wood |
| Strings | Painted |
| Handrail | Maple or Oak |
| Balusters | Maple or Oak |
| Notes |  |

| 20b. STAIRS: |  |
| --- | --- |
| Tread |  |
| Risers |  |
| Strings |  |
| Handrail |  |
| Balusters |  |

| 21. SPECIAL FLOORS AND WAINSCOT: |  |  |
| --- | --- | --- |
| Tile floors | Master Bath, Bath #3, Bath #4, Utility |  |
| Tile walls Tile Tub Surround | Master Bath Shower – Porcelain Hampton 12”x12” Master Bath Room – Porcelain Hampton 12”x12” |  |
| Crown Moldings Notes Living Room |  |  |
| Chair Rails | N/A |  |
| Other trim specialties | 4” Stain Grade Trim @ Ceiling – Office; Stain Grade Trim @ Soffit in Dining, Hearth and Media Rooms |  |
| Specialty allowances |  |  |

| 22. PLUMBING: |  |
| --- | --- |

| GENERAL |  |
| --- | --- |
| Hose bibs | 3 |
| water heater: | 80 gal Whirlpool electric, located in Utility Room Supplied by Solar Consultants |
| floor drains: |  |
| drain system: | Schedule 40 PVC |
| supply system: | PEX Tubing |
| sewage disposal | On site, perk field |
| water supply | On site, well |

PLUMBING SCHEDULE

| ROOM | SINKS | COMMODE(S) | MIXERS | SHOWER | TUBS | MISCELLANEOUS |
| --- | --- | --- | --- | --- | --- | --- |
| Powder Rm | Soapstone w/ Delta 2530 Chrome Mixer | Elger – El Bowl 1.5 Gallon Flush |  |  |  | White |
| Kitchen | Double Stainless w/ Delta 2276 Mixer |  |  |  |  | Ice Maker Hook-Up |
| Laundry | Wall hung |  |  |  |  | Washer Hook-Up |
| Bath -Master | Two cultured marble w/ Delta 520 Chrome Mixers | Elger - El Bowl 1.5 Gallon Flush |  | Poreclain Tile –Hampton 12x12 Delta 621 Chrome Mixer | Kohler Oval 70”x42” Soaker w/Delta 2780 | Poreclain tile –Hampton 12x12 Tub Surround |
| Bath # 2 Bath # 3 | Two cultured marble w/ Delta 520 Chrome Mixers Cultured marble w/ Delta 520 Chrome Mixer | Elger - El Bowl 1.5 Gallon Flush Elger-El Bowl 1.5 Gallon Flush |  | Molded Fiberglass Tub/ Shower w/ Delta 635 Chrome Mixer Molded Fiberglass Shower w/ Delta 621 Chrome Mixer |  | Molded Fiberglass |
| Exterior |  |  |  |  |  | Three Hose Bibs |

| 23. HEATING |  |
| --- | --- |
| HEATING SYSTEMS |  |
| Dual Fuel Heat pump | High Efficiency Dual Fuel Heat Pump (LP Gas as backup to an electric heat Pump) with a High SEER AC Unit. |
| COOLING SYSTEMS: |  |
| Central A/C system | Condensor unit as required |
| VENTILATING EQUIPMENT |  |
| Bath vents | FanTech roof mounted upstairs, Nutone in powder Rm; Nutone in Master; electronic timers |
| Dryer duct | Ducted to outside |
| Kitchen vents: Whole House Fan | Vent a Hood @ cooktop, ducted to outside N/A |
| DUCT MATERIALS |  |
| Trunk | Rigid |
| Branch | Flex |
| Insulation |  |
| ADDITIONAL INFORMATION |  |

| 24. ELECTRIC WIRING |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PANEL: |  |  |  |  |  |  |  |  |
| Make |  |  |  |  |  |  |  |  |
| AMP's |  |  | 1 – 200A main; 1 – 100A sub panel |  |  |  |  |  |
| NO. circuits |  |  |  |  |  |  |  |  |
| WIRING: |  |  | Romex, unless otherwise indicated |  |  |  |  |  |
| SPECIAL OUTLETS: |  |  |  |  |  |  |  |  |
| Cook top |  |  | X |  |  |  |  |  |
| Oven |  |  | X |  |  |  |  |  |
| Water heater |  |  | X |  |  |  |  |  |
| Clothes dryer |  |  | X |  |  |  |  |  |
| Micro wave |  |  | X |  |  |  |  |  |
| Dishwasher |  |  | X |  |  |  |  |  |
| Refrigerator |  |  | X |  |  |  |  |  |
| Doorbell |  |  | X – Two chime |  |  |  |  |  |
| A/C Units (describe) |  |  | X disconnect @ each unit |  |  |  |  |  |
| Sewer Pump |  |  | As required |  |  |  |  |  |
| ADDITIONAL INFORMATION: |  |  |  |  |  |  |  |  |
| 25. LIGHTING FIXTURES: $2500 allowance for 3 outside fixtures 1 outside fan/light on porch 1 dining room fixture 4 bath vanity fixtures 2 foyer sconces 6 fan light in bedrooms |  |  | FILL IN SCHEDULE BELOW OR ATTACH SCHEDULE FROM SUPPLIER(S) |  |  |  |  |  |
| ROOM | FIXT 1 | FIXT 2 |  | FIXT 3 | FIXT 4 | FIXT 5 | FIXT 6 |  |

| 26. INSULATION |  |
| --- | --- |
| Roof | 9” R-30 |
| Ceiling | 9” R-30 |
| Walls | 6” R 19 |
| Floor Bonus Room Floor | 6” R-19 6” R-19 |

| 27. MISCELLANEOUS: |  |
| --- | --- |
| Appliances | BOSCH SHE ss 4 program dishwasher GE J2s968 shss 30” slide in dual fuel range/oven GE JE2160SH2.1cf ss microwave GE microwave trim kit Vent a Hood prh9130ss 300 cfm hood |
| Special Hardware allowances | SEE ATTACHED SHEET |

| 28. HARDWARE: |  |
| --- | --- |
| Front Door |  |
| Towel Bars |  |
| Soap Dishes |  |
| Mirrors Shower Door |  |
| ALLOWANCE for All Hardware | $1,600.00 |

| 29. PORCHES & DECKS: |  |
| --- | --- |
| Front Rear Screened Porch | Per Plan with 4/4 ipe decking. Ceiling- Bead Board N/A Per Plan with 4/4 ipe decking. Per Plan. Bead Board Ceiling, 4/4 ipe decking. Code Permitting 30” Knee Walls Covered with Bead Board Panels. Posts 6x6 Pressure Treated. |

| 30. OUTBUILDINGS | None |
| --- | --- |

| 31. WALKS AND DRIVEWAYS: |  |  |
| --- | --- | --- |
| Drive |  |  |
| Width | 10’ |  |
| Length | TBD with Turn Around in Front of Garage |  |
| Surface | concrete |  |
| ENTRY STEPS: |  |  |
| Treads | P.T. SYP |  |
| Risers | “ |  |
| Rails | “ |  |
| REAR STEPS: |  |  |
| Treads | P.T. SYP |  |
| Risers | “ |  |
| Rails | “ |  |
| FRONT WALKWAY: |  |  |
| Length | TBD |  |
| Material | Poured Concrete or Concrete Pavers Set in a Mortar Bed |  |

| PATIOS |  |
| --- | --- |

| 32. OTHER ONSITE IMPROVEMENTS |  |  |
| --- | --- | --- |
| Plantings |  |  |
| Retaining walls |  |  |
| Terraces |  |  |
| Improvement allowance |  |  |
| Other |  |  |

| 33. LANDSCAPING, PLANTING, AND FINISH GRADING: |  |  |
| --- | --- | --- |
| Seeded areas | All Disturbed Ground |  |
| Mulched Areas | All Disturbed Ground |  |
| Trees and Shrubs | $1500 |  |
| Other |  |  |
| Landscaping allowance |  |  |

| 34. ITEMIZED ALLOWANCES |  |
| --- | --- |
| CABINETS/TOPS BUILT INS |  |
| STONE WORK - INTERIOR And EXTERIOR |  |
| APPLIANCES | SEE ATTACHED SHEET |
| CARPET | $30/yard Installed |
| CLOSET/PANTRY SYSTEMS | $2,000 |
| LIGHT FIXTURES | $2,500 |
| LANDSCAPING | $1500 |
| Bath HARDWARE, MIRRORS SHWR. DOOR CABINET PULLS | $1,600 $2/Pull |
| WELL DRILLING | Well Supplied |
| SEPTIC SYSTEM | Septic System Supplied |

SPECIAL EQUIPMENT:

It is important to make special note of the solar nature of this house. It is designed to qualify for all three segments of Federal and North Carolina renewable energy Income Tax Credits: 1) PV electric power systems; 2) solar hot water collection systems; and 3) passive solar design.

Federal solar tax incentives enacted in the Energy Policy Act of 2005 allow homeowners a tax credit of 30% for qualifying solar electric (PV) or solar water heating expenditures, up to a maximum of $2,000 per technology. For businesses, the investment tax credit is increased from 10% to 30% of qualified solar property expenditures, with no cap on the credit amount. These incentives apply to equipment placed in service during 2006-2007.

This house will come with a 1920 watt PV solar electric power system. BP Solar (British Petroleum) panels or equivalent. Inverter  by Trace  #SW 4048 or equivalent.  The system will have the capability of supplying utility grade current to the grid, supplying utility grade current to the “emergency” sub panel, and managing the integral 5000 watt battery powered back-up power system. The system is fully automated.  This system will enable the homebuyer to participate in the NC Green Power program, and realize approximately $600 of revenue from selling the PV generated power into the power grid. During times when the utility power grid is not functioning, the system automatically switches over to the battery powered backup power source to keep critical functions, such as well, refrigerator, lighting, and gas furnace operational. The installed cost of this system will be approximately $18,000. The NC Energy Income Tax Credit of 35%, as applied to this segment,  generates a credit for the home buyer of  $6,300.00

The house will contain a Solargenix CPC 2000 hot water heating system. It is capable of providing approximately 80% of the annual hot water heating requirements for the house. The Solargenix system couples with a standard electric hot water heater. This solar hot water collection system is self-contained, and thus places no additional demands upon the solar power system in the house, and uses stock componentry inside the house. There are two roof mounted, integrated collector panels, and one small PV power panel. The installed cost of this system will be approximately $3,500.00. The NC Energy Income Tax Credit of 35%, as applied to this segment, generates a credit for the homebuyer of  $1,225.00.

The house incorporates solar tempered design, as defined by the NC Solar Income Tax credit language. The design satisfies the criteria for: 1) the ratio of south facing glazing relative to the living space floor; 2) properly sized overhangs; 3) open space design; and 4) and proper site orientation. The NC Energy Income Tax Credit allows for a maximum credit, related to design, of $3,500.00.

Furthermore, the house will participate in the Energy Star program, as designed and directed by the U.S. Department of Energy.  This high performance, energy efficiency building program is supported and recognized by Progress Energy Corporation, who will provide the electrical service for the homes at Bingham Ridge.  There are participation fees of approximately $750.00 to enroll in the Energy Star program, and additional construction related costs of approximately $5,500.00. These costs are not currently applicable to the N.C. Renewable Energy Income Tax Credit.  Statistics support that building to the specifications of this program and offering the enhanced energy features to the buyer adds value for both the buyer and the lender.

| 35. PAINTING: Three (3) Wall Colors Included in Contract $150 Each Wall Additional Color |  |
| --- | --- |

36. FINISH

SCHEDULE

| ROOM | WALLS | CEILING | FLOOR | CROWN | BASE | CASINGS |
| --- | --- | --- | --- | --- | --- | --- |

37.  PAINT SCHEDULE  To be determined

| ROOM | WALLS | CEILING | FLOOR | CROWN | BASE | CASINGS |
| --- | --- | --- | --- | --- | --- | --- |

BUYER___________________________________________________________________

DATE_______________________________________________________________________

BUYER________________________________________________________________________

DATE_____________________________________________________________________________

CONTRACTOR_________________________________________________________________________

DATE____________________________________________________________________________________